Customer SDK

Secondary-Development API Guide

Open-layer APIs · Scenario guides · AI constraints

Product secondary development only

Suggested order: Getting started → (optional) AI-assisted → G00 → G01 → technical chapters as needed

Authoritative definitions: include/*.h and main/app/*.h. Do not reverse-engineer closed .a libraries.

Table of Contents

(In Word: References → Table of Contents; or select the field below and press F9)

Table of Contents	2

0. How do I start secondary development?	6

1. Mental model	6

2. Suggested learning path (by day)	6

3. Three valid approaches	7

4. Never do this	8

5. Next steps	8

AI-Assisted Secondary Development	9

1. Why feed the docs to the AI	9

2. Recommended customer workflow	9

3. Files to attach (by scenario)	10

4. Prompt templates (copy-paste)	10

5. Red-line checklist (human or AI review)	11

6. How the manual is structured for AI	12

7. Cursor tips	12

8. Related links	13

AI_CONTEXT — Customer SDK constraint card (for AI)	14

Project	14

Architecture (mandatory)	14

Allowed edits	14

Forbidden	14

Preferred APIs	14

RS485 defaults (configurable)	15

Doc index (read before implementing)	15

Output rules (for the AI)	15

Language	15

G00. Secondary-development directions (multi-tech fusion)	16

Overview	16

Direction 1 — On-panel Modbus controller (best fit for “RS485 + LVGL”)	16

Direction 2 — Cloud smart panel	17

Direction 3 — Near-field BLE / Mesh	17

Direction 4 — Ethernet + RS485 gateway	17

Direction 5 — Whole-home scene hub	17

Direction 6 — Low-power info / read-only meter	18

Direction 7 — OEM theme reskin	18

How to choose	18

Shared architecture	18

G01. Walkthrough: Modbus + RS485 + LVGL device control	20

1. Data flow (understand before coding)	20

2. Hardware and Kconfig	20

3. Implementation steps	21

4. Example register map (replace with your device manual)	23

5. Acceptance checklist	23

6. API quick reference	23

G02. Scenario: Wi-Fi + MQTT + Hub cloud panel	25

User story	25

Technologies	25

Steps	25

Notes	25

See also	25

G03. Scenario: BLE / Mesh near-field control	26

User story	26

Technologies	26

Steps	26

See also	26

G04. Scenario: Ethernet + RS485 gateway	27

User story	27

Technologies	27

Steps	27

See also	27

G05. Scenario: whole-home multi-channel hub	28

User story	28

Technologies	28

Recommended flow	28

Design rules	28

See also	28

G06. AI prompt cheat sheet	29

1. Session opener (recommended every time)	29

2. Short prompts by direction	29

3. Ask the AI to review	30

4. Commit message help	30

1. Overview & layers	31

Layers	31

Doc map	31

2. Environment & build	32

3. Boot order	33

4. Wi-Fi	34

5. MQTT (Bemfa)	35

6. Bluetooth BLE / Mesh	36

7. Ethernet	37

8. Backlight	38

9. Buzzer	39

10. GPIO_OUT / RS485 (detailed)	40

1. Role	40

2. Hardware defaults	40

3. API reference	41

4. Threading (required)	41

5. Kconfig	41

6. Troubleshooting	42

11. Modbus RTU (detailed)	43

1. Dependencies	43

2. API	43

3. Correct LVGL / Hub wiring	44

4. Error codes	44

5. Extending	44

12. RF coexistence	45

13. Low power	46

14. Cross-thread GUI (detailed)	47

1. API	47

2. Standard pattern	47

3. Relation to bus work	48

15. Hub model & UI (detailed)	49

1. Core headers	49

2. Common model APIs	49

3. Theme secondary development	50

4. Fusion with other channels	50

16. Icons	51

17. Health monitor	52

18. Kconfig	53

19. Control map	54

20. FAQ	55

Appendix A: Header index	56

Open layer (prefer for secondary development)	56

Closed library public headers	56

■  Getting started

0. How do I start secondary development?

▎ This page answers: I have Customer SDK and want LVGL + RS485/Modbus device control — what do I change, and how do the pieces connect?

────────────────────────

1. Mental model

The firmware is a on-screen state machine plus several control channels:

┌─────────────┐ Touch / theme UI ►│ hub_model │◄── MQTT / BLE / custom tasks └──────┬──────┘ │ hub_ui_refresh() ▼ LVGL UI │ (optional) bus bridge task ▼ app_rs485 / app_modbus_* ──► field devices

| You want to change | Edit here | Do not |
| --- | --- | --- |
| Colors / layout | ui/themes/<theme>/ | Closed .a libraries |
| On/off semantics | hub_model_* APIs | Invent a parallel state store |
| Bus I/O | main/app/app_rs485* / app_modbus* | Block UART reads in LVGL callbacks |
| Boot order | main/main.c | Break NVS → display → network order |

Common includes:

#include "app_api.h" /* RS485 / Modbus / beep / coexist */ #include "hub_model.h" #include "hub_ui.h" #include "gui_task.h"

────────────────────────

2. Suggested learning path (by day)

| Stage | Goal | Doc |
| --- | --- | --- |
| Day 0 | Build, flash, icons + touch OK | Build |
| Day 1 | Theme colors / one toast button | Hub model & UI |
| Day 2 | Cross-thread label updates | GUI task |
| Day 3 | RS485 TX/RX works | GPIO_OUT / RS485 |
| Day 4 | Modbus read holding registers | Modbus RTU |
| Day 5 | LVGL button → Modbus write | G01 guide |
| Day 6+ | Pick a fusion direction | G00 directions |

Using AI tools? → AI-assisted development (pin AI_CONTEXT first).

────────────────────────

3. Three valid approaches

A — UI / theme only (fastest)

Set APP_UI_THEME_ID in main/app_ui_theme_select.h

Edit ui/themes/<id>/palette.c, theme_local.c, pages_*.c

In callbacks call only hub_model_* + hub_ui_refresh()

Best for reskins and copy changes.

B — UI + bus bridge (typical: Modbus lights / curtains)

app_rs485_init (watch UART0 vs console conflict)

Add main/app/app_bus_bridge.c: queue of “UI intents”, worker calls app_modbus_write_single

Theme callback: hub_model_set_widget_level → enqueue → optional MQTT sync

Full steps: G01. Skeleton: main/app/examples/.

C — Multi-channel product

The same hub_model is updated from:

Touch UI

Bemfa MQTT

BLE apply_set_state

RS485/Modbus poll-back

Rule: after any path changes device state, update `hub_model` and refresh the UI. See Control map and G00.

────────────────────────

4. Never do this

while-wait for Modbus replies inside LVGL event callbacks

Skip hub_model and only paint labels (cloud/BLE will desync)

Call esp_wifi_init again / reinvent the display stack

Depend on testbench ui_testbench_* (validation project, not product APIs)

────────────────────────

5. Next steps

AI-assisted coding → AI_DEV.md

RS485 devices only → G01

Cloud panel → G02

More product shapes → G00

AI-Assisted Secondary Development

▎ When using Cursor, ChatGPT, Copilot, or similar tools on this SDK, prepare context and prompts as described here. That sharply reduces mistakes such as editing closed .a libraries or blocking Modbus inside LVGL callbacks.

────────────────────────

1. Why feed the docs to the AI

This SDK is intentionally layered:

| You may change | Do not change / invent |
| --- | --- |
| main/, ui/themes/, spiffs_image/, main/app/app_*.c | Internals of components/*/lib/**/*.a |
| Public header APIs | Testbench esp32_s3_frame ui_testbench_* (not product APIs) |
| Examples under main/app/examples/ | Full pin maps (unpublished; use app_rs485_config_t / board APIs) |

If the model only skims source without docs, it often:

Calls app_modbus_write_single directly from a button callback (freezes touch)

Bypasses hub_model and only updates lv_label

Tries to edit or reverse .a files, or invents non-existent RS485 headers

Correct approach: make the AI read the listed docs and headers first, then state the task.

────────────────────────

2. Recommended customer workflow

① Pick a direction (G00) → ② Open the matching guide (e.g. G01) → ③ Attach AI_CONTEXT + related markdown/headers → ④ Use a prompt template from this page → ⑤ Human review: open layer only? thread-safe? → ⑥ Validate on hardware

| Step | You | AI |
| --- | --- | --- |
| Goal | Choose G01–G05 | — |
| Context | @ docs and headers | Learn constraints |
| Plan | Paste a template | File-level change plan |
| Code | Confirm the plan | Edit only main/ / ui/themes/ |
| Review | Red-line checklist | Fix per your feedback |

────────────────────────

3. Files to attach (by scenario)

3.1 Always useful

| File | Why |
| --- | --- |
| docs/api_guide/en/AI_CONTEXT.md | One-page constraint card for AI |
| docs/api_guide/en/00_secondary_dev.md | Mental model |
| main/app/app_api.h | Open API umbrella |
| components/hub_core/include/hub_model.h | Device-state API |

3.2 Modbus + RS485 + LVGL (most common)

Also attach:

docs/api_guide/en/guides/G01_modbus_rs485_lvgl.md

docs/api_guide/en/10_gpio_rs485.md

docs/api_guide/en/11_modbus.md

docs/api_guide/en/14_gui_task.md

main/app/app_rs485.h, app_modbus_rtu.h

main/app/examples/app_bus_bridge_example.c

3.3 Cloud / BLE / gateway

| Direction | Also attach |
| --- | --- |
| MQTT cloud panel | G02, wifi_bemfa_client.h, wifi_management.h |
| BLE | G03, bt_management.h, app_coexist.h |
| Ethernet gateway | G04, board_ethernet_ch390.h |
| Whole-home fusion | G05, G00 |

In Cursor use @file / @folder. In ChatGPT paste AI_CONTEXT.md plus the relevant chapters.

────────────────────────

4. Prompt templates (copy-paste)

Template A — On-panel Modbus light control

You are a secondary-development assistant for the ESP32-S3 + LVGL Customer SDK. Obey the red lines in docs/api_guide/en/AI_CONTEXT.md. Goal: a touch toggle for the living-room light writes Modbus slave 1, register 0 (0/1) over RS485. Requirements: 1. Follow G01 and main/app/examples/app_bus_bridge_example.c 2. LVGL/theme callbacks only update hub_model + enqueue; Modbus runs in a FreeRTOS worker 3. Edit only main/ and ui/themes/; do not modify .a files or invent pin macros 4. Deliver: file list, CMake changes, main.c hook, theme callback sample Output a change plan first; wait for my confirmation before writing code.

Template B — Theme UI only

Use the Customer SDK Hub theme model (see 15_hub.md). Goal: change a home-page button label to "Leave home"; on click call apply_scene("away") and hub_ui_refresh. Edit only files under ui/themes/<current theme>/. Do not touch bus or networking. First name which file and callback you will change.

Template C — Bus not responding

Customer SDK: app_rs485_init(NULL) done; modbus_read_holding fails. Console may still own UART0. Using 10_gpio_rs485.md / 20_faq, give a checklist (DE, A/B, baud, console conflict, slave address). Do not suggest reverse-engineering board_bsp.a.

Template D — MQTT + same bus bridge

Assume the G01 bus bridge already exists. Make Bemfa MQTT on/off commands share app_bus_bridge_post_write with touch. Read G02, G05, wifi_bemfa_client.h. Name the extension points. Never block network or Modbus on the LVGL thread.

Template E — Force docs-first answers

Before answering, list the repo doc paths you will rely on (must include AI_CONTEXT.md). If docs conflict with generic knowledge, prefer this repo. Do not use esp32_s3_frame ui_testbench_* APIs.

────────────────────────

5. Red-line checklist (human or AI review)

| # | Check | Pass criteria |
| --- | --- | --- |
| 1 | Scope | Only main/, ui/themes/, spiffs_image/, docs |
| 2 | State | Device state via hub_model_*; refresh via hub_ui_refresh / gui_task_post_lvgl |
| 3 | Bus threads | No long read/transact/modbus_* inside LVGL callbacks |
| 4 | APIs | Includes from app_*.h / public components/*/include |
| 5 | Closed libs | No .a edits; no undeclared private symbols |
| 6 | UART | Console vs RS485 UART assignment documented |
| 7 | Example | Bus bridge based on examples/app_bus_bridge_example.c |

────────────────────────

6. How the manual is structured for AI

| Doc type | AI use |
| --- | --- |
| AI_CONTEXT.md | System / pinned constraints |
| 00_secondary_dev.md | Architecture |
| guides/G*.md | Task playbooks |
| 10/11/14/15_*.md | API detail and anti-patterns |
| A_headers.md | Prevent invented include paths |
| examples/*.c | Low-hallucination code start |

Maintenance principles:

Hard-code paths for @ mentions

Pair good/bad examples

Scenario steps usable as AI task text (G01-level)

Constraints first in AI_CONTEXT, not buried at the end

────────────────────────

7. Cursor tips

Use Chat for plans, Agent for file edits; plan before large diffs

Start with @docs/api_guide/en/AI_CONTEXT.md + @guides/G01...

Require: “list files to change; never modify components/*/lib”

After generation, ask the model to self-check against section 5

For “no slave response”, check FAQ first so the model does not invent pins

────────────────────────

8. Related links

| Doc | Role |
| --- | --- |
| AI_CONTEXT.md | Constraint card (pin first) |
| G06 prompts | Short prompt cheat sheet |
| Secondary development | Human overview |
| G00 directions | Product shapes |
| G01 walkthrough | Modbus + LVGL |
| Chinese edition | ../zh/AI_DEV.md |

AI_CONTEXT — Customer SDK constraint card (for AI)

▎ Pin this file at the top of the conversation. When it conflicts with generic ESP-IDF advice, prefer this file and the English API guide in this folder.

Project

Name: ESP32-S3 Customer SDK (smart panel / Hub UI)

Target: esp32s3 · ESP-IDF 5.5.x

Project root: customer_sdk/ (do not randomly edit a parent private monorepo)

Architecture (mandatory)

UI / MQTT / BLE ──► hub_model ──► hub_ui_refresh / gui_task_post_lvgl │ └─ (optional) queue ──► FreeRTOS task ──► app_rs485 / app_modbus_*

Single source of device truth: hub_model_* (components/hub_core/include/hub_model.h)

Open peripheral entry: #include "app_api.h" (main/app/)

Cross-thread LVGL updates: must use gui_task_post_lvgl (never call lv_* from other tasks)

Allowed edits

main/** (including main/app/, main/hub_ui/, main/main.c)

ui/themes/**

spiffs_image/**

docs/**

Forbidden

Editing or reverse-engineering components/*/lib/**/*.a

Treating esp32_s3_frame ui_testbench_* as product APIs

Long blocking calls in LVGL event callbacks: app_rs485_read / transact / app_modbus_* / network I/O

Bypassing hub_model and only changing widget text for “device state”

Calling esp_wifi_init again or reinventing RGB display bring-up

Inventing unpublished GPIO macros or a full expander pin map

Preferred APIs

| Need | API |
| --- | --- |
| RS485 | app_rs485_init/write/read/transact |
| Modbus | app_modbus_read_holding / write_single |
| Beep | app_beep_pulse / click_if_enabled |
| Coexistence | app_coexist_before/after_ble_scan |
| UI state | hub_model_* + hub_ui_refresh |
| Bus example | main/app/examples/app_bus_bridge_example.c |

RS485 defaults (configurable)

UART0 · TX43 · RX44 · 115200 · DE = app_gpio_out_set

If the console still owns UART0: switch to USB-JTAG or pass a custom app_rs485_config_t

Doc index (read before implementing)

| Task | Doc |
| --- | --- |
| Overview | docs/api_guide/en/00_secondary_dev.md |
| AI workflow | docs/api_guide/en/AI_DEV.md |
| Modbus + LVGL | docs/api_guide/en/guides/G01_modbus_rs485_lvgl.md |
| Directions | docs/api_guide/en/guides/G00_directions.md |
| Headers | docs/api_guide/en/A_headers.md |

Output rules (for the AI)

First list “files I will modify” and “paths I will not touch”

Wait for confirmation before large code dumps (unless the user asks to edit immediately)

Code must separate queue/worker I/O, or explicitly reuse the example bridge

Self-check against the red-line table in AI_DEV.md

Language

Answer in the user’s language; code comments may be English or Chinese.

■  Guides

G00. Secondary-development directions (multi-tech fusion)

▎ This chapter lists shippable product shapes. Each direction combines technologies already in this SDK. Pick one, then follow the matching guide.

────────────────────────

Overview

| Direction | Technologies | Difficulty | Guide |
| --- | --- | --- | --- |
| ① On-panel Modbus controller | LVGL + Hub + RS485 + Modbus + beep | ★★☆ | G01 |
| ② Cloud smart panel | LVGL + Wi-Fi + MQTT + Hub + ambient backlight | ★★☆ | G02 |
| ③ BLE / Mesh local control | LVGL + BLE/Mesh + Hub + coexist | ★★★ | G03 |
| ④ Wired gateway | Ethernet + RS485/Modbus + Hub ops page | ★★★ | G04 |
| ⑤ Whole-home scene hub | ①+②+③, one hub_model | ★★★★ | G05 |
| ⑥ Low-power info display | Ambient + backlight + Wi-Fi/SNTP + sensors | ★★☆ | See “Direction 6” below |
| ⑦ OEM reskin | Themes + SPIFFS icons | ★☆☆ | Hub, Icons |

Using AI? Pin AI_CONTEXT and read AI_DEV.

────────────────────────

Direction 1 — On-panel Modbus controller (best fit for “RS485 + LVGL”)

Story: Tap “Living room light” on the screen; Modbus over RS485 writes a slave register; the light turns on and the UI stays in sync.

Button → hub_model_toggle → queue → bus worker → app_modbus_write_single → slave → on success: hub_ui_refresh + app_beep_pulse

You will use: hub_ui / hub_model, gui_task, app_rs485, app_modbus_rtu, app_beep, app_coexist (ETH SPI paused during bus writes).

→ Details: G01

────────────────────────

Direction 2 — Cloud smart panel

Story: Phone (Bemfa) and the touch panel share the same hub_model; idle timeout dims the backlight.

MQTT command → (library) → hub_model Touch → hub_model → wifi_bemfa_client_schedule_sync()

You will use: wifi_management, wifi_bemfa_client, hub_model, UI_AMBIENT_*.

Can stack with direction 1: cloud commands also go through the same app_bus_bridge.

→ G02

────────────────────────

Direction 3 — Near-field BLE / Mesh

Story: Mini-program or provisioner sends SET_STATE; the panel updates; optional Modbus write-out.

BLE SET_STATE → bt_management_apply_set_state (you override) → hub_model + optional Modbus

Before scans call app_coexist_before_ble_scan() so CH390 SPI does not fight the radio.

→ G03

────────────────────────

Direction 4 — Ethernet + RS485 gateway

Story: Ethernet uplink (TCP/HTTP you add); Modbus RTU downlink; ops page shows link health.

ETH got IP → custom TCP service → parse → app_modbus_* hub_model()->protos[] updated for ETH / RS485 health

→ G04

────────────────────────

Direction 5 — Whole-home scene hub

Story: One “Home” scene: UI updates, MQTT sync, BLE notify neighbors, batch Modbus writes.

Shared rules:

Scenes only call hub_model_apply_scene

One bridge task owns all bus writes

No channel paints LVGL directly—only the model

→ G05

────────────────────────

Direction 6 — Low-power info / read-only meter

Story: Hallway panel shows time and temperature; after idle it dims; occasional Wi-Fi sync.

Tech: app_low_power_init, UI_AMBIENT_*, board_backlight_set, AHT20 (ui_bg_task), optional Ethernet receive-only.

RS485 can stay off; orthogonal to direction 1.

────────────────────────

Direction 7 — OEM theme reskin

Edit only ui/themes/ + spiffs_image/icons; business still uses hub_model. Fast for branding.

────────────────────────

How to choose

| Your goal | Pick |
| --- | --- |
| “No cloud yet—just drive RS485 lights” | Direction 1 |
| “Need a phone app” | Direction 2 (+ optional 1) |
| “Field near-field debug” | Direction 3 |
| “Machine room has Ethernet” | Direction 4 |
| “Full central panel product” | Direction 5 |
| “Mostly display, little interaction” | Direction 6 |
| “New UI skin only” | Direction 7 |

────────────────────────

Shared architecture

Open (editable): ui/themes/* look & feel main/hub_ui/* shell (change carefully) main/app/app_*.c bus / beep / coexist main/main.c boot hooks Closed (headers only): hub_model / wifi_* / bt_* / board_*

Next: for physical devices open G01 Modbus + RS485 + LVGL. For AI-assisted edits open AI_DEV and pin AI_CONTEXT.

G01. Walkthrough: Modbus + RS485 + LVGL device control

▎ Goal: Use the touch UI on this SDK to control Modbus slaves on RS485 (lights, curtains, relays, …).   Audience: You can already idf.py flash, but are unsure where button-press code should go.

────────────────────────

1. Data flow (understand before coding)

┌──────────────┐ ① mutate model ┌────────────┐ │ LVGL button │ ───────────────────► │ hub_model │ └──────┬───────┘ └─────┬──────┘ │ ② enqueue (non-blocking) │ ③ hub_ui_refresh() ▼ ▼ ┌──────────────┐ ┌────────────┐ │ FreeRTOS q │ │ UI paint │ └──────┬───────┘ └────────────┘ │ ④ worker dequeues ▼ ┌──────────────┐ ⑤ DE+UART ┌────────────┐ │ app_modbus_* │ ────────────► │ RS485 slave│ └──────┬───────┘ └────────────┘ │ ⑥ success / fail ▼ app_beep / update_hub_health / (optional) MQTT sync

Critical: steps ①② on the LVGL thread are “mutate model + enqueue” only; step ⑤ must run in a dedicated task.

────────────────────────

2. Hardware and Kconfig

2.1 Default pins (WithTheWind-validated)

| Signal | Default |
| --- | --- |
| UART | UART0 |
| TX / RX | GPIO43 / GPIO44 |
| DE/RE | Expander GPIO_OUT (high = transmit) |
| Baud | 115200 8N1 |

Confirm with your hardware vendor before production; override via app_rs485_config_t.

2.2 Console conflict

If CONFIG_ESP_CONSOLE_UART_NUM=0, UART0 fights RS485. Choose one:

menuconfig → console = USB Serial/JTAG (recommended)

Or set app_rs485_config_t to UART_NUM_1 + other TX/RX pins

2.3 menuconfig

Secondary-dev peripherals [*] APP_ENABLE_RS485 [ ] APP_RS485_AUTO_INIT ← keep off until UART conflict is resolved

────────────────────────

3. Implementation steps

Step A — Prove the bus without UI

After display init in app_main, temporarily:

#include "app_api.h" app_rs485_init(NULL); uint16_t reg = 0; esp_err_t e = app_modbus_read_holding(0x01, 0x0000, 1, &reg, 1, 800); ESP_LOGI("demo", "modbus read: %s val=%u", esp_err_to_name(e), reg);

Only wire UI after this succeeds. Failures: DE polarity, A/B wiring, slave address, baud, console conflict.

Step B — Intent queue (recommended files)

Copy:

main/app/examples/app_bus_bridge_example.c → main/app/app_bus_bridge.c

Matching header → app_bus_bridge.h

Add the .c to MAIN_SRCS in main/CMakeLists.txt

Job shape:

typedef enum { BUS_CMD_WRITE_HOLDING = 1, BUS_CMD_READ_HOLDING, } bus_cmd_t; typedef struct { bus_cmd_t cmd; uint8_t slave; uint16_t addr; uint16_t value; /* write */ int room, slot; /* for UI feedback */ } bus_job_t;

Worker loop:

for (;;) { bus_job_t job; if (xQueueReceive(q, &job, portMAX_DELAY) != pdTRUE) continue; esp_err_t err = ESP_FAIL; if (job.cmd == BUS_CMD_WRITE_HOLDING) { err = app_modbus_write_single(job.slave, job.addr, job.value, 800); } app_rs485_update_hub_health(err == ESP_OK, err == ESP_OK ? 95 : 20, true); if (err == ESP_OK) { app_beep_pulse(60); } else { hub_model_toast("Bus write failed"); gui_task_post_lvgl(refresh_cb, NULL); } }

Step C — LVGL / theme callbacks only enqueue

In theme_local.c or a room-page button:

#include "hub_model.h" #include "hub_ui.h" #include "app_bus_bridge.h" #include "app_beep.h" static void on_light_toggle(lv_event_t *e) { (void)e; const int room = 0, slot = 0; hub_model_toggle_widget(room, slot); hub_ui_refresh(); app_beep_click_if_enabled(); hub_widget_t *w = hub_model_widget_by_slot(room, slot); if (w == NULL) return; /* Convention: slave 1, reg 0 = on/off 0/1 */ app_bus_bridge_post_write(1, 0x0000, w->on ? 1 : 0); }

Do not call app_modbus_write_single here.

Step D — Optional poll-back

Every ~2 s in a worker:

uint16_t v = 0; if (app_modbus_read_holding(1, 0, 1, &v, 1, 500) == ESP_OK) { hub_model_set_widget_level(0, 0, v ? 100 : 0); gui_task_post_lvgl(refresh_cb, NULL); }

Keeps the screen aligned if someone toggles the field switch manually.

Step E — Optional MQTT fusion

After a successful local write:

wifi_bemfa_client_publish_status_u8(item_id, value, true); wifi_bemfa_client_schedule_sync();

Cloud-originated commands must call the same app_bus_bridge_post_write so phone / panel / bus stay consistent.

────────────────────────

4. Example register map (replace with your device manual)

| Hub widget | Slave | FC | Addr | Meaning |
| --- | --- | --- | --- | --- |
| Living light ONOFF | 1 | 06 | 0x0000 | 0=off 1=on |
| Dimmer | 1 | 06 | 0x0001 | 0–100 |
| Curtain | 2 | 06 | 0x0010 | 0–100% |

Keep a static widget → (slave, addr) table inside app_bus_bridge.c so themes do not hard-code magic numbers.

────────────────────────

5. Acceptance checklist

[ ] read_holding succeeds with no UI

[ ] Touch toggle produces a visible write on scope / slave

[ ] Touch stays smooth (bus I/O off the LVGL thread)

[ ] Unplugging the bus shows toast / lower protocol health

[ ] (Optional) Phone MQTT matches panel state

────────────────────────

6. API quick reference

| Need | API |
| --- | --- |
| Init bus | app_rs485_init |
| Write register | app_modbus_write_single |
| Read register | app_modbus_read_holding |
| Panel device state | hub_model_toggle_widget / set_widget_level |
| Refresh UI | hub_ui_refresh or gui_task_post_lvgl |
| Beep feedback | app_beep_pulse / click_if_enabled |
| Protocol slot | app_rs485_update_hub_health |

Details: RS485 · Modbus · Hub · gui_task

Example sources: main/app/examples/app_bus_bridge_example.c

G02. Scenario: Wi-Fi + MQTT + Hub cloud panel

User story

A phone (Bemfa cloud) and the touch panel share the same hub_model device state. After idle, the panel dims into ambient mode.

Technologies

wifi_management · wifi_bemfa_client · hub_model / hub_ui · UI_AMBIENT_* · board_backlight_* · (optional) bus bridge → Modbus

Steps

Enable APP_FEATURE_MQTT=y and join Wi-Fi from the network page

Confirm MQTT connected in the log

After local changes:

hub_model_toggle_widget(room, slot); hub_ui_refresh(); wifi_bemfa_client_schedule_sync();

If RS485 is also used: cloud and touch must share app_bus_bridge_post_write (G01)

Ambient: UI_AMBIENT_ENABLE + TIMEOUT_SEC + BACKLIGHT_PCT

Notes

Default UID/topics live in the closed library; customize via vendor or your own MQTT client

Do not block on network I/O inside LVGL callbacks

See also

Wi-Fi · MQTT · Low power

G03. Scenario: BLE / Mesh near-field control

User story

A mini-program or provisioner sends SET_STATE nearby; the panel updates; optionally the same command is written to a Modbus slave.

Technologies

bt_management · bt_proto · app_coexist · hub_model · (optional) RS485

Steps

Enable APP_FEATURE_BLE / MESH as needed

Override the weak symbol:

esp_err_t bt_management_apply_set_state(uint8_t item_id, uint8_t value) { /* Map item_id → room/slot */ hub_model_set_widget_level(0, 0, value ? 100 : 0); hub_ui_refresh(); app_bus_bridge_post_write(1, 0x0000, value); /* optional */ return ESP_OK; }

Around BLE scans:

app_coexist_before_ble_scan(); bt_management_ble_scan_start(5); /* … */ app_coexist_after_ble_scan();

See also

Bluetooth · Coexistence · G01

G04. Scenario: Ethernet + RS485 gateway

User story

The panel has Ethernet uplink; you add TCP/HTTP northbound; Modbus RTU southbound; the ops page shows ETH / RS485 health.

Technologies

board_ethernet_ch390_* · app_coexist · app_rs485 / app_modbus_* · hub_model.protos[] · gui_task

Steps

Confirm board_ethernet_ch390_init + try_start and a valid get_ip

Before heavy ETH work: app_coexist_before_ethernet_work()

In open main/, add a TCP server task; after parse call app_modbus_write_single

Periodically update protocol health:

bool eth_ok = board_ethernet_ch390_link_up(); /* update hub_model()->protos[i] */ app_rs485_update_hub_health(rs485_ok, health, true);

app_rs485_write already pauses CH390 SPI during bursts

See also

Ethernet · RS485 · Coexistence

G05. Scenario: whole-home multi-channel hub

User story

One “Home” scene: UI updates, cloud sync, BLE notify, and batch Modbus actions to several slaves.

Technologies

Directions 1+2+3: hub_model_apply_scene is the only scene entry point.

Recommended flow

hub_model_apply_scene("home") │ ├─► hub_ui_refresh() ├─► wifi_bemfa_client_schedule_sync() ├─► bt_management_mesh_send_set_state(...) /* optional */ └─► queued batch of bus_jobs └─► app_modbus_write_single × N

Design rules

Single truth: trust only hub_model

Single bridge: all bus writes go through one app_bus_bridge

Single refresh path: non-LVGL threads use gui_task_post_lvgl

Visible failures: toast + protos[].health

See also

G00 directions · Control map · G01

G06. AI prompt cheat sheet

Use with AI-assisted development and AI_CONTEXT.md. Replace … with your concrete goal.

────────────────────────

1. Session opener (recommended every time)

Please read and obey @docs/api_guide/en/AI_CONTEXT.md . This repo is Customer SDK — edit open layers only. Do not use ui_testbench APIs; do not modify .a files.

────────────────────────

2. Short prompts by direction

Modbus light control

Follow @docs/api_guide/en/guides/G01_modbus_rs485_lvgl.md , integrate examples/app_bus_bridge_example into the project, theme callback: living-room light toggle → post_write(1,0,on). List files first.

Theme reskin only

Edit only ui/themes and spiffs_image. See @docs/api_guide/en/15_hub.md and @docs/api_guide/en/16_icons.md . Goal: …

MQTT + same bus

bus_bridge already exists. Read G02/G05 so cloud on/off shares post_write with touch.

BLE point

Override bt_management_apply_set_state, update hub_model, optional post_write. Wrap scans with app_coexist_before/after_ble_scan. See G03.

Build / link errors

Analyze the log; prefer A_headers and CMakeLists; do not suggest editing lib/*.a contents.

────────────────────────

3. Ask the AI to review

Review my latest diff against AI_DEV.md section 5 red lines. List violations and fixes only (do not rewrite unrelated files).

────────────────────────

4. Commit message help

From the current diff, write a 1–2 sentence English commit message focused on why (e.g. add Modbus bridge so the panel can drive lights).

■  Technical

1. Overview & layers

▎ This guide covers secondary development of the Customer SDK. For “LVGL + Modbus + RS485 device control”, start with:   0. Getting started → G01 walkthrough

Layers

| Layer | Path | Notes |
| --- | --- | --- |
| Open application | main/, ui/themes/, spiffs_image/ | Editable source |
| Open peripheral APIs | main/app/app_*.h | RS485, Modbus, beep, coexist, … |
| Examples | main/app/examples/ | Bus-bridge skeleton (add to build yourself) |
| Closed libraries | components/*/include + .a | Wi-Fi / BT / Hub / board |

#include "app_api.h"

Doc map

| Need | Entry |
| --- | --- |
| How to develop secondarily | 00 |
| Product directions (7 fusions) | G00 |
| Technical API chapters | 02–20 in this folder |

────────────────────────

2. Environment & build

ESP-IDF 5.5.x · target esp32s3

idf.py build / flash must include the SPIFFS storage partition

cd customer_sdk idf.py set-target esp32s3 idf.py build idf.py -p COMx flash monitor

If RS485 uses UART0: switch the console to USB Serial/JTAG (matches validated test setup), or override pins via app_rs485_config_t.

────────────────────────

3. Boot order

wifi_management_foundation_init(); app_low_power_init(); bt_management_init(...); /* optional */ app_spiffs_mount(); board_display_start_with_lvgl_cfg(...); board_ethernet_ch390_init/try_start(); board_backlight_init(); /* optional: app_rs485_init(NULL) — Kconfig APP_RS485_AUTO_INIT */ app_ui_start(); gui_task_init(); wifi_management_start(); bt_management_start(); /* optional */

I2C / expander become available only after display init. RS485 DE (GPIO_OUT) depends on that order.

────────────────────────

4. Wi-Fi

#include "wifi_management.h" #include "app_wifi_util.h"

| API | Role |  |  |
| --- | --- | --- | --- |
| wifi_management_foundation_init/start | Foundation / STA |  |  |
| wifi_management_connect/disconnect_user | Connect / disconnect |  |  |
| wifi_management_scan_blocking | Blocking scan |  |  |
| wifi_management_is_connected | Has IP |  |  |
| app_wifi_scan_sorted | Scan sorted by RSSI |  |  |
| app_wifi_format_status | `"SSID \ | IP \ | RSSI"` |

wifi_ap_record_t aps[16]; uint16_t n = 16; app_wifi_scan_sorted(aps, &n); wifi_management_connect("MySSID", "pwd");

Do not block for long scans inside LVGL callbacks—run them from a worker or network page task.

────────────────────────

5. MQTT (Bemfa)

#include "wifi_bemfa_client.h"

| API | Role |
| --- | --- |
| wifi_bemfa_client_start/stop | Start/stop (often auto-starts on GOT_IP) |
| wifi_bemfa_client_publish_status_u8 | Single-point reply |
| wifi_bemfa_client_schedule_sync | Full snapshot sync |

Requires APP_FEATURE_MQTT=y. After local state changes: hub_model_* → hub_ui_refresh() → schedule_sync().

When also driving RS485, cloud commands must share the same bus bridge as touch (G01 · G02).

────────────────────────

6. Bluetooth BLE / Mesh

#include "bt_management.h" #include "bt_proto.h" #include "app_coexist.h"

Before scans:

app_coexist_before_ble_scan(); bt_management_ble_scan_start(5); /* … done … */ app_coexist_after_ble_scan();

Point control: provide a non-weak bt_management_apply_set_state(item_id, value) that updates hub_model and calls hub_ui_refresh(). Optionally enqueue the same Modbus write as the panel (G03).

────────────────────────

7. Ethernet

#include "board_ethernet_ch390.h" #include "app_coexist.h"

| API | Role |
| --- | --- |
| board_ethernet_ch390_init/try_start | Init / start |
| get_ip / link_up / get_link_info | Status |
| set_traffic_paused | Pause SPI (used by app_coexist_*_rs485_burst during RS485 bursts) |

Before heavy ETH work: app_coexist_before_ethernet_work().

Gateway pattern: G04.

────────────────────────

8. Backlight

#include "withthewind_board_lvgl_init.h" #include "hub_device_ui.h"

Call board_backlight_init, then after the first frame board_backlight_on / board_backlight_set(0..100). Settings page helpers: hub_device_brightness_get / hub_device_brightness_apply_effective.

Idle dimming is usually driven by UI_AMBIENT_* (see Low power).

────────────────────────

9. Buzzer

#include "app_beep.h" /* low-level */ #include "withthewind_board_lvgl_init.h" /* board_beep_set */

| API | Role |
| --- | --- |
| board_beep_set(on) | Direct on/off |
| app_beep_pulse(ms) | One-shot pulse (esp_timer) |
| app_beep_pulse_n(n, on, off) | Multi-beep |
| app_beep_click_if_enabled() | Respects hub_model click-sound setting |

app_beep_pulse(120); app_beep_click_if_enabled();

Safe to call from UI callbacks; pulses do not depend on LVGL timers.

────────────────────────

10. GPIO_OUT / RS485 (detailed)

▎ Half-duplex RS485 is the transport layer for field devices in this SDK. Modbus and custom protocols sit on top of it.

────────────────────────

1. Role

| API group | Role |
| --- | --- |
| app_gpio_out_* | Expander GPIO_OUT; used as RS485 DE/RE on this board |
| app_rs485_* | UART half-duplex: DE high → TX → wait done → DE low → RX |

Headers:

#include "app_rs485.h" #include "app_gpio_out.h" #include "board_io_expander.h" /* safe subset */

────────────────────────

2. Hardware defaults

| Item | Default | Notes |
| --- | --- | --- |
| UART | UART_NUM_0 | Pair with USB-JTAG console |
| TX / RX | GPIO 43 / 44 | WithTheWind-validated |
| DE | app_gpio_out_set(true) to transmit | High = TX, low = RX |
| Baud | 115200 8N1 | Overridable via config |

app_rs485_config_t cfg; app_rs485_get_default_config(&cfg); cfg.uart_num = UART_NUM_1; /* example: avoid console UART */ cfg.tx_gpio = /* confirm with hardware vendor */; cfg.rx_gpio = /* ... */; cfg.baud = 9600; app_rs485_init(&cfg);

────────────────────────

3. API reference

| Function | Description |
| --- | --- |
| app_rs485_init(cfg) | Install driver; cfg==NULL uses defaults; idempotent |
| app_rs485_is_ready() | Whether init succeeded |
| app_rs485_write(data,len,timeout) | Half-duplex write; pauses ETH SPI internally |
| app_rs485_read(buf,cap,wait_ms) | Timed read; returns byte count or negative error |
| app_rs485_transact(...) | Write then read reply |
| app_rs485_update_hub_health(ok,health,refresh) | Update ops-page protocol slot |
| app_rs485_deinit() | Release UART |

DE timing (inside write):

DE=1 → delay de_setup_ms → uart_write → wait_tx_done → delay turnaround → DE=0

────────────────────────

4. Threading (required)

| Thread | Allowed |
| --- | --- |
| LVGL / button callbacks | Only xQueueSend intents — no long read / transact |
| Your bus worker task | write / read / modbus_* |
| UI feedback | gui_task_post_lvgl or update_hub_health(..., true) |

Full light-control flow → G01

────────────────────────

5. Kconfig

| Option | Default | Meaning |
| --- | --- | --- |
| APP_ENABLE_RS485 | y | Compile this module |
| APP_RS485_AUTO_INIT | n | app_main auto-init (enable only after UART conflict is resolved) |

────────────────────────

6. Troubleshooting

| Symptom | Check |
| --- | --- |
| TX but no reply | DE polarity, A/B wiring, termination, slave address |
| Init garbles log / crashes | UART0 vs console conflict → JTAG or another UART |
| Touch stutters | Bus I/O ran on the LVGL thread |

Next: Modbus RTU

11. Modbus RTU (detailed)

▎ Thin client on top of app_rs485: function codes 03 read holding and 06 write single register. Enough for most lights / relays / modules; extend FC yourself for complex devices.

────────────────────────

1. Dependencies

app_modbus_* ──depends──► app_rs485_* ──depends──► UART + DE(GPIO_OUT)

#include "app_modbus_rtu.h" #include "app_rs485.h"

Call app_rs485_init before first use (read_holding may also attempt init).

────────────────────────

2. API

| Function | FC | Description |
| --- | --- | --- |
| app_modbus_crc16(data,len) | — | Standard Modbus CRC16 |
| app_modbus_read_holding(slave,addr,qty,out,out_n,timeout_ms) | 0x03 | Read holding registers |
| app_modbus_write_single(slave,addr,value,timeout_ms) | 0x06 | Write single register |

Read example

uint16_t regs[4]; esp_err_t err = app_modbus_read_holding( 0x01, /* slave */ 0x0000, /* start register */ 2, /* quantity */ regs, 4, 700); if (err == ESP_OK) { ESP_LOGI("mb", "r0=%u r1=%u", regs[0], regs[1]); }

Write example

/* turn relay on: write 1 to address 0 */ esp_err_t err = app_modbus_write_single(0x01, 0x0000, 0x0001, 700);

Probe frame (same as testbench): 01 03 00 00 00 01 84 0A

────────────────────────

3. Correct LVGL / Hub wiring

Wrong:

void btn_cb(lv_event_t *e) { app_modbus_write_single(...); /* blocks touch! */ }

Right: see G01 — callback only mutates hub_model + enqueues; Modbus runs in a worker.

Mapping tips:

| Widget type | Typical register meaning |
| --- | --- |
| ONOFF | 0/1 |
| DIMMER / CURTAIN | 0–100 |
| CLIM setpoint | temp ×10 etc. (per device manual) |

────────────────────────

4. Error codes

Common returns: ESP_OK, ESP_ERR_TIMEOUT, ESP_ERR_INVALID_CRC, ESP_ERR_INVALID_RESPONSE, ESP_ERR_INVALID_SIZE. On failure call app_rs485_update_hub_health(false, …) so the ops page shows it.

────────────────────────

5. Extending

For FC05/0F/10: build frames with app_modbus_crc16 + app_rs485_transact, or bring in esp-modbus on the same UART (do not mix with this thin client concurrently).

Next: RF coexistence · Hands-on: G01

12. RF coexistence

#include "app_coexist.h"

| API | When |
| --- | --- |
| app_coexist_before/after_ble_scan | Around BLE scans |
| app_coexist_before_ethernet_work | Before heavy Ethernet traffic |
| app_coexist_before/after_rs485_burst | RS485 bursts (app_rs485_write already calls these) |
| app_coexist_heap_ok_for_eth | DMA-heap gate for ETH |

CH390 shares SPI with other activity; pausing SPI during RS485 / coordinating around BLE scans prevents link drops and garbled bus frames.

────────────────────────

13. Low power

#include "app_low_power.h" app_low_power_init(); /* after foundation, before display */

On RGB panels, Light sleep is typically not enabled by default. For idle power saving, use UI_AMBIENT_* to dim the backlight instead of deep sleep.

See also: Backlight · direction 6 in G00.

────────────────────────

14. Cross-thread GUI (detailed)

▎ LVGL is not thread-safe. Wi-Fi events, Modbus workers, and MQTT callbacks must not call lv_label_set_text directly—post work to the LVGL thread.

────────────────────────

1. API

#include "gui_task.h" gui_task_init(); /* already called after UI in main.c */ gui_task_post_lvgl(cb, user_data); gui_task_post_lvgl_ex(cb, user_data, fail_free); /* free heap if queue full */ gui_task_notify_lvgl(cb, user_data); /* lightweight path */

────────────────────────

2. Standard pattern

typedef struct { lv_obj_t *label; char text[48]; } ui_msg_t; static void apply_cb(void *arg) { ui_msg_t *m = arg; lv_label_set_text(m->label, m->text); free(m); } void from_bus_task(const char *status) { ui_msg_t *m = malloc(sizeof(*m)); m->label = s_status_label; snprintf(m->text, sizeof(m->text), "%s", status); if (!gui_task_post_lvgl_ex(apply_cb, m, free)) { /* fail_free already ran on queue-full; check impl if false */ } }

Refreshing the whole Hub page is simpler:

static void refresh_cb(void *arg) { (void)arg; hub_ui_refresh(); } gui_task_post_lvgl(refresh_cb, NULL);

app_rs485_update_hub_health(..., true) already goes through gui_task.

────────────────────────

3. Relation to bus work

Bus task ──success/fail──► gui_task_post_lvgl ──► hub_ui_refresh / toast LVGL callback ──only──► hub_model + QueueSend (never wait on the bus)

Details: G01.

15. Hub model & UI (detailed)

▎ hub_model is the single source of truth for on-screen device state. Themes only decide “how it looks”; on/off semantics always go through model APIs.

────────────────────────

1. Core headers

| Header | Use |
| --- | --- |
| hub_model.h | Rooms / widgets / scenes / settings / toast |
| hub_ui.h | hub_ui_init/go/refresh, routing |
| hub_theme.h | Theme hooks |
| hub_device_ui.h | Network page, brightness |
| hub_icons.h | SPIFFS icons |

────────────────────────

2. Common model APIs

hub_model_t *m = hub_model(); hub_model_set_room(0); hub_model_apply_scene("home"); hub_model_toggle_widget(room, slot); hub_model_step_widget(room, slot, +5); hub_model_set_widget_level(room, slot, 80); hub_model_toast("Done"); hub_ui_refresh(); /* refresh current page after model changes */

| Type hub_wtype_t | Typical use |
| --- | --- |
| HUB_W_ONOFF | Light / relay |
| HUB_W_DIMMER | Dimmer |
| HUB_W_CURTAIN / SHUTTER | Curtain / shutter |
| HUB_W_CLIM / THERMO | AC / floor heat |
| HUB_W_PLUG / FAN | Plug / fan |

────────────────────────

3. Theme secondary development

Switch theme: main/app_ui_theme_select.h → APP_UI_THEME_ID

Colors: ui/themes/<id>/palette.c

Layout: home.c / pages_*.c

Must implement: hub_theme_build(parent, route)

Button callback template (when wired to the bus):

hub_model_toggle_widget(room, slot); hub_ui_refresh(); app_beep_click_if_enabled(); app_bus_bridge_post_write(slave, addr, on ? 1 : 0); /* see examples */

────────────────────────

4. Fusion with other channels

| Channel | How to update UI |
| --- | --- |
| Touch | Direct hub_model_* + hub_ui_refresh |
| Modbus poll task | Mutate model + gui_task_post_lvgl(refresh) |
| MQTT / BLE | Mutate model + refresh + (optional) bus write-out |

Related: Getting started · G01 · G00

16. Icons

Replace PNGs under spiffs_image/icons/nt/*.png, then idf.py flash (SPIFFS storage must be flashed).

APIs: app_spiffs_mount, hub_ico_cache_init, hub_ico_add.

Blank icons usually mean storage was not flashed, or POSIX FS / PNG path mismatch — see FAQ.

────────────────────────

17. Health monitor

#include "app_health.h" app_health_monitor_start();

Optional heap / system health task (APP_HEALTH_MONITOR). Protocol health on the ops page is updated separately via app_rs485_update_hub_health and hub_model()->protos[].

────────────────────────

18. Kconfig

| Option | Meaning |
| --- | --- |
| APP_FEATURE_MQTT / BLE / MESH | Cloud / Bluetooth features |
| APP_ENABLE_RS485 | Compile RS485 / Modbus APIs |
| APP_RS485_AUTO_INIT | Call app_rs485_init at boot (default n) |
| UI_AMBIENT_* | Idle backlight / ambient mode |
| APP_HEALTH_MONITOR | Heap / health monitor |

Configure with idf.py menuconfig under the Customer SDK / secondary-dev menus. Keep APP_RS485_AUTO_INIT off until UART console conflict is resolved (RS485).

────────────────────────

19. Control map

All channels should converge on `hub_model`, then optionally write the bus and/or sync to the cloud.

| Entry | Suggested flow |
| --- | --- |
| Touch UI | hub_model_* → (optional) app_bus_bridge_post_write → Modbus |
| Bemfa MQTT | In-library points → same bridge write-out → schedule_sync |
| BLE SET_STATE | apply_set_state → model → same bridge |
| Ethernet TCP | Your parser → app_modbus_* → refresh model |
| One-tap scene | apply_scene → batched queue writes |

Touch / MQTT / BLE / ETH / Scene │ ▼ hub_model ←── single truth │ ┌────────┼────────┐ ▼ ▼ ▼ hub_ui bus_bridge MQTT sync refresh Modbus schedule_sync

Pick a product shape → G00 Panel → RS485 walkthrough → G01

────────────────────────

20. FAQ

| Problem | What to do |
| --- | --- |
| Blank icons | Flash storage (SPIFFS); confirm PNG paths + POSIX FS |
| RS485 no reply | Check DE polarity; console vs UART0; app_rs485_is_ready() |
| Missing expander symbols at link | Customer subset restored in board_io_expander.h; ensure board_bsp.a is linked |
| No beep on press | Use app_beep_click_if_enabled or check click-sound in hub_model |
| Touch freezes when controlling devices | Move Modbus/RS485 off the LVGL thread — see G01 |
| ETH drops during Modbus | Ensure writes go through app_rs485_write (coexist pause) |
| AI suggests editing .a / ui_testbench | Reject — pin AI_CONTEXT |

────────────────────────

Appendix A: Header index

Open layer (prefer for secondary development)

| Feature | Header |
| --- | --- |
| Umbrella include | main/app/app_api.h |
| Beep pulse | main/app/app_beep.h |
| Coexistence | main/app/app_coexist.h |
| GPIO_OUT / DE | main/app/app_gpio_out.h |
| RS485 | main/app/app_rs485.h |
| Modbus | main/app/app_modbus_rtu.h |
| Wi-Fi helpers | main/app/app_wifi_util.h |
| Low power | main/app/app_low_power.h |
| GUI post | main/gui/gui_task.h |
| Expander (safe subset) | components/board_bsp/include/board_io_expander.h |

Closed library public headers

| Feature | Header |
| --- | --- |
| Wi-Fi | wifi_management.h |
| MQTT | wifi_bemfa_client.h |
| Bluetooth | bt_management.h / bt_proto.h |
| Ethernet | board_ethernet_ch390.h |
| Backlight / beep HW | withthewind_board_lvgl_init.h |
| Hub model | hub_model.h |
| Hub UI | hub_ui.h |

Example (not in default build): main/app/examples/app_bus_bridge_example.h

────────────────────────